Layout of the model

Wheel Base: 292.10mm

Wheel Track: 193.02mm

Overall Length: 388.07mm

Overall width: 220.07mm

Overall height: 94.04mm

Front Steering servo

Wheel Motor

Wheel Motor

Wheel Motor

Battery

Power Hub

ESC-Electronic Speed Controller

Wheel Motor

Sketch of the circuit of the power assembly

Wheel Motor

JST-ZH 6Pin Connector

3.5 mm Gold Connector

Switch

Wheel Motor

JST-ZH 6Pin Connector

3.5 mm Gold Connector

Switch

Power Hub

4 mm Gold Connector

Wheel Motor

JST-ZH 6Pin Connector

3.5 mm Gold Connector

Switch

When reconnect the brushless motor’s 3.5 mm gold connectors, make sure to match the wires with the same color (Black wire is wrapped by the color shrink tube at the connector).

Wheel Motor

JST-ZH 6Pin Connector

3.5 mm Gold Connector

Switch

Servo

- The wire definition of the servo cables of the typical brands. The servo on the prototype 1 is the HITEC: HS-5685MH.

- Input to the servo.

  - 4.8v-7.4v DC for power supply

  - 4.8v-7.4v 50Hz Square wave signal input

- Output of the servo.

  - 11.3 kg·cm, 0.2 sec/60°at 6v power supply.

  - 12.9 kg·cm, 0.17sec/60°at 7.4v power supply.

Steering system

Steering angle sensor

- Since a 1:5 gear reduction is added to the servo, the output of the steering system,

  - 56.5 kg·cm, 1 sec/60°at 6v power supply.

  - 64.5 kg·cm, 0.85sec/60°at 7.4v power supply.

- Control signal

  - 50Hz(20ms) square wave

  - Center position: 7.5 % (1.5ms), Positive 45°: 10 % (2ms), Negative 45°: 5% (1ms).

  - Rotation limit: ±30°- duty cycle limit: 5.83% (1.165ms) ~ 9.17% (1.835ms).

  - The holes can be used as reference if you need to calibrate the PWM signal.

22.5 °

ESC – Electronic Speed Controller

- A 6v DC-DC is embedded into the ESC.

  - The DC-DC is originally used for powering the receiver and servos.

  - When you have an external power supply, don’t connect the red wire to it.

  - If you use one PWM signal to control multiple ESC, don’t connect red wires together.

- Signal

  - The same signal requirement as the servo.

  - Neutral: 7.5 % (1.5ms), forward: 7.5% (1.5ms) ~ 10 % (2ms), reverse: 7.5% (1.5ms) ~ 5% (1ms).

  - Electronic brake is applied when signal is switched from forward to reverse. When the signal is switched from reverse to forward, the motor will change the direction immediately and drain large current, avoid this situation.

  - The reverse will only be enabled when signal stays on the neutral for 2 second.

Hall-Effect sensors port

To motor

Ground

DC-DC output

Signal

Battery in

Switch

Hall-Effect sensors

- The sensor is Honeywell SS360NT SOT-23 Bipolar latch hall effect sensor. More information is on its data sheet.

- Since the rotor has 7 pairs of magnets, the triggering period of a single sensor is every 360/7≈51.43°, and the sensor switches output every 360/7/2≈25.715°.

- The three sensor is evenly located on a circle within an angle of 360/7≈51.43°, which means 360/7/3≈17.14°between adjacent ones.

Actual color is different, just refer the number.

Pin5 is not used.

Connect to a servo tester

Other references

http://www.techmonkeybusiness.com/calibrating-the-rovs-escs.html

NVIDIA Jetson TX1

GPIO Library Repo:

https://github.com/NVIDIA/jetson-gpio

# IDIA Jetson TX1 J21 Header Pinout

| Sysfs GPIO | Name | Pin | Pin | Name | Sysfs GPIO |
| --- | --- | --- | --- | --- | --- |
|  | 3.3 VDC Power | 1 | 2 | 5.0 VDC Power |  |
|  | SDA1 General I2C #0 Data 3.3.V, I2C Bus 0 | 3 | 4 | 5.0 VDC Power |  |
|  | SCL1 General I2C #0 Data 3.3.V, I2C Bus 0 | 5 | 6 | GND |  |
| gpio216 | GPIO_GCLK AUDIO_MCLK Audio Master Clock (1.8/3.3.V) | 7 | 8 | TXD0 UART #0 Transmit |  |
|  | GND | 9 | 10 | RXD0 UART #0 Receive |  |
| gpio162 | GPIO_GEN0 UART #0 Request to Send | 11 | 12 | GPIO_GEN1 Audio I2S #0 Clock | gpio11 |
| gpio38 | GPIO_GEN2 Audio Code Interrupt | 13 | 14 | GND |  |
| gpio511 | GPIO_GEN3 From GPIO Expander (P17) | 15 | 16 | GPIO_GEN4 Unused | gpio37 |
|  | 3.3 VDC Power | 17 | 18 | GPIO_GEN5 Modem Wake AP GPIO | gpio184 |
| gpio16 | SPI_MOSI SPI #1 Master Out/Slave In | 19 | 20 | GND |  |
| gpio17 | SPI_MISO SPI #1 Master In/Slave Out | 21 | 22 | GPIO_GEN6 From GPIO Epander (P16) | gpio510 |
| gpio18 | SPI_SCLK SPI #1 Shift Clock | 23 | 24 | SPI_CE0_N SPI #1 Chip Select #0 | gpio19 |
|  | GND | 25 | 26 | SPI_CE1_N SPI #1 Chip Select #1 | gpio20 |
|  | ID_SDA General I2C #1 Data (3.3V), I2C Bus 1 | 27 | 28 | ID_SCL General I2C #1 Clock (3.3V), I2C Bus 1 |  |
| gpio219 | GPIO5 Audio Reset (1.8/3.3V) | 29 | 30 | GND |  |
| gpio186 | GPIO6 Motion Interrupt (3.3V) | 31 | 32 | GPIO12 Unused | gpio36 |
| gpio63 | GPIO13 AP Wake Bt GPIO | 33 | 34 | GND |  |
| gpio8 | GPIO19 AUDIO I2S #0 Left/Right Clock | 35 | 36 | GPIO16 UART #0 Clear to Send | gpio163 |
| gpio187 | GPIO26 (3.3V) | 37 | 38 | GPIO20 Audio I2S #0 Data in | gpio9 |
|  | GND | 39 | 40 | GPIO21 Audio I2S #0 Data in |  |